Source:	Fraunhofer IIS

Title:	Thoughts on IVAS Objective Testing and Test Database

Document for:	Discussion

Agenda Item:	1.3

The IVAS Design Constraints (IVAS-4, [1]) contain objective requirements on JBM performance or Output Gain Tolerance. Further on, for former standardization activities (e.g. EVS), additional objective requirements, such as VAD performance, were tested. These objectives tests are not really covered yet in the permanent documents. In addition, the source would like to bring to the attention that one or several objective test databases are required.

This contribution starts with looking what was done for EVS and develops next steps for the IVAS standardization from there.

- EVS – Objective Testing

Objective Evaluations performed for EVS

The objective evaluations performed for EVS during selection testing are listed in EVS-8b [2], Annex M:

“

1. Gain verification (design constraints) (Gain)

2. JBM compliance to TS 26.114 (design constraints) (JBM)

3. AFR (performance requirements) (AFR)

4. Attenuation during inactive regions (performance requirements) (Att.)

5. Average active speech bit rate of VBR and CBR (BR)

6. Complexity measurement (Cmp)

“

On order to perform these tests, in total 15 databases were created, consisting of

- NB, WB, SWB clean speech

- NB, WB, SWB noisy speech (car noise)

- NB, WB, SWB noisy speech (street noise)

- NB, WB, SWB noisy speech (office noise)

- NB, WB, SWB mixed/music

The creation of the databases is described in EVS-8a [3], Annex H:

- The clean speech database was assembled by a volunteer and consist of speech samples in different languages. Approximately 40 percent of the clean speech databases consists of active speech. It was assembled from sentences using the ITU-T P.501 and ITU-T P.501 Amd1, the speech portion of the ETSI STQ database, and the common corpus that is collected from proponents under the multi-party NDA. The database had a length of >15 minutes.

- The databases with background noises were assembled by a volunteer and were generated by mixing the clean speech database with the background noise items that were accepted. The databases had a length of >15 minutes each.

- The database for mixed content and music consists of all items that have been submitted by the proponents for the subjective listening tests and were accepted. A volunteer has assembled the common database from all accepted items. The databases had a length of >45 minutes.

EVS-8b, section M3 describes the exact usage of the databases for the various objective tests and are summarized here:

- Gain verification: Clean speech, noisy speech, mixed/music databases

- JBM compliance to TS 26.114: Clean speech, mixed/music databases

- AFR: Noisy speech database

- Attenuation during inactive regions: Clean speech, noisy speech databases

- Average active speech bit rate of VBR and CBR: Clean speech, noisy speech, mixed/music databases

- Complexity measurement: Clean speech, noisy speech, mixed/music databases

- IVAS Objective Testing – Discussion

Although details on the objective testing are still missing from IVAS-3 permanent document [4], it feels urgent to start thinking about test databases for the objective testing. Compared to EVS, it also needs to be stated that there is a lack of available immersive audio material from previous standardization efforts which could be re-used.

The source would like to raise the following points for discussion:

- IVAS supports the following input formats: Channel-based audio, binaural audio, SBA, MASA, object-based audio. It seems to be logical that different databases per input are required. However, it is unclear to the source whether these separate databases can be generated based on a common origin (e.g. all derived from a HOA3 baseline or from mono sources + corresponding model), which would simplify the item collection and provide more consistent results across input formats.

- The latest version of IVAS-8a distinguishes between two scenarios: a) Immersive conversation use case scenario and b) Generic immersive audio use case scenario. It is unclear to the source whether this impacts the objective databases. From a functionality perspective, the approach from EVS, which distinguishes between clean speech, noisy speech and mixed/music seems to be clear and straight-forward.

- A sufficient amount of audio samples has to be collected so that databases of sufficient length can be established. Long enough databases are crucial to test e.g. JBM, AFR, bitrate and complexity. The EVS databases had a length of minimum 15 minutes each. It is currently unclear whether this much material is publicly available and possible to be used for the standardization effort or whether it needs to be collected e.g. by the proponent companies (as partially for EVS). In case it needs to be collected, it needs to be checked whether a legal framework is needed for this.

- Conclusion

The source proposes to discuss and decide on the points raised in section 2.

References

[1] S4-221619 – IVAS Design Constraints (IVAS-4)

[2] S4-141319 - EVS Permanent Document EVS-8b: Test plans for selection phase including lab task specification

[3] S4-130127 - EVS Permanent Document EVS-8a: Test plans for qualification phase including host lab specification

[4] S4-221525 - IVAS Performance Requirements (IVAS-3)

[5] S4aA230005 – IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.5.1