Source:	Nokia

Title:	Single source scenario for performance evaluation

Document for:	Discussion & Agreement

Agenda Item:	1.7 – DaCAS

# 1. Introduction

The source proposes revised recording scenario for objective assessment of single source capture. The proposed modifications are based on the received comments during the last SA4-132 meeting and further to allow sufficient flexibility for all proponents with different recording facilities/capabilities.

# 2. Recording scenario for single source capture evaluation

In [1], a brief overview on available objective performance evaluation methodologies according to the 3GPP TS 26.260 [2] was presented. Based on the overview, a proposal on applied recording scenario for aforesaid assessment was presented in [3]. Based on the received feedback, further adaptations and modifications for the proposed recording scenario on evaluating DaCAS example solutions are proposed in the Table 1 below. Aim of the presented adaptations is to allow reasonable flexibility while maintaining sufficient control on the applied scenario. Presented recording scenario is proposed to be included to the DaCAS-2 permanent document.

The relaxations in the proposed recording scenario considers namely the requirements of the applied sound source and the requirements of the utilized acoustic environment. Concerns were raised that some of the specified loudspeaker requirements are still provisional in the 3GPP TS 26.260. It is considered that high quality loudspeaker which doesn’t comply with clause 4.0.2 in 3GPP TS 26.260 may be also used with sufficient calibration. However, it is recommended to use loudspeaker that complies with the provisional values.

In addition, as all proponents may not have access to the anechoic chamber, also other acoustically well treated rooms are proposed to be acceptable. It should be kept in mind, that such relaxation may have impact on the obtained performance characteristics.

While two capture modes are defined in TS 26.260 (user and spatial capture), it is proposed that for DaCAS evaluation only spatial capture mode is considered. This simplifies the recording scenario and furthermore reduce the number of required recordings. Once the recording scenario is agreed, it is proposed that all the proponents would conduct required recordings according to the agreed recording scenario.

Table 1: Recording scenario for single sound source scenario

| No | X (Single source objective test) |
| --- | --- |
| Sound source | High quality loudspeaker Recommended to use loudspeaker that complies with clause 4.0.2 in 3GPP TS 26.260 (including provisional values). |
| Source signal | British English single talk sequence according to the ITU-T P.501 |
| Source signal characteristics | Language: English Gender: Male, Female Range: 20Hz-20kHz Length: 35.4 s Level: -27 dB RMS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration | Loudness: Source signal playback calibrated to 75 dB SPL according to the clause 5.5.1 of 3GPP TS 26.260. Frequency response: The spectrum of the acoustic signal produced by the loudspeaker shall be equalized with a measurement microphone positioned on the main loudspeaker axis at 1 m from the loudspeaker membrane. The achieved equalized spectrum in 1/3rd octave bands, when measured in the test environment, shall be within ±1 dB from 100 Hz to 200 Hz and shall be within ±0.5 dB from 200 Hz to 20 kHz. |
| Acoustic environment | Anechoic chamber OR acoustically well treated room with short reverberation (room compliant e.g., with Clause 6.1. in ETSI TS 103 224, or with clause 8 in ITU-T BS.1116). Recommended environment is an anechoic chamber according to the clause 4.0.3 of 3GPP TS 26.260. |
| Detailed Positioning | Source distance: - For Hand-held and Headset devices 1 – 1.5 m - For table-mounted devices/table orientation of the device, source placement is according to the clause 5.4.2.5 for Spatial capture arrangement in 3GPP TS 26.260 Source direction: Loudspeaker diaphragm towards the UE Source angles: Azimuth angles: 0°, ±30°, ±60°, ±90° - For Hand-held and Headset devices elevation angle 0° - For table-mounted devices elevation angle 26.6° (see clause 5.4.2.5 in 3GPP TS 26.260) |
| Reference recording |  |
| Description/additional info | More information can be found from 3GPP TS 26.260 Applied sound source, acoustic environment, and mounting of the device shall be clearly documented. |

# 3. Summary

In this document a further revision on applied recording scenario for objective assessment of DaCAS solution single source capture is proposed. Proposed changes are mainly on relaxation of the sound source and acoustic environment requirements. The proposed recording scenario in Table 1 is proposed to be agreed and integrated to the DaCAS-2.

# References

[1]	S4aA250021: “On objective performance evaluation”

[2]	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive audio systems” version 18.1.0 Release 18

[3]	S4-250957: “On performance evaluation”, Nokia