Source: 	InterDigital Inc.

Title: 	examples of Display Capabilities

Document for	Discussion and agreement

Agenda item: 	3.3 - MeCAR (Media Capabilities for Augmented Reality)

# Introduction

This contribution proposes to enrich the permanent document with examples of display capabilities as they have been listed in the TR 26.998 and as they are implemented on a real product.

# Examples of display capabilities

TR26.998 [1] in clause 4.5.2 has gathered a collection of features and KPIs focusing on AR glasses. These features detail, among others, display capabilities. For the sake of illustration, it proposed to reproduce these examples of display capabilities in a table here below.

In addition, real products of optical see-through devices are starting to emerge. We refer here to the Nreal light AR glasses, which may be considered as one of the most advanced implementation of optical see-through AR glasses. In order to enrich the illustration, a left column is added with the corresponding characteristics of the Nreal light AR glasses.

Specifications of Nreal light AR glasses can be found here : https://www.nreal.ai/specs/.

A review of the Nreal light AR glasses can be found here : https://www.tomshardware.com/reviews/nreal-light-ar-smart-glasses.

| Display capability | KPIs for AR glasses (Table 4.5.2-1 from TR26.998, clause 5.4.2) | Nreal Light (optical see-through) |
| --- | --- | --- |
| Persistence – Duty time | Turn pixels on and off every 2 - 4 ms to avoid smearing / motion blur | No data |
| Display refresh rate | - 60 Hz minimum - 90 Hz acceptable - 120 Hz and beyond desired - 240 Hz would allow always on display at 4ms | - 60Hz |
| Colour | - RGB colours - Accurate colours independent of viewpoint. | - RGB, 106% color gamut - Color depth 8 bits for 16.773 million colors - OLED display |
| Spatial Resolution Per Eye | For 30 x 20 degrees - 1.5K by 1K per eye is required - 1.8K by 1.2K per eye is desired For 40 x 40 degrees - 2K by 2K required - 2.5 K by 2.5 K desired Ultimate goal for display resolution is reaching or going slightly beyond the human vision limit of roughly one arcmin (1/60°) | - 52° FoV - 42 pixels per degree - 1920x1080 per eye |
| Brightness | - 200-500 nits for indoor - Up to 2K for state-of-the-art devices in 2021 [49] - 10K to 100K nits for full outdoor experiences | - Up to 280 nits perceived brightness. |
| Field of View | Augmentable FoV - typically, 30 by 20 degrees FoV acceptable - 40 by 40 degrees desired maximize the non-obscured field-of-view | - 52° |

# References

- TR 26.998 “Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices”.

- Nreal Glasses Technical Specifications, Online: https://www.nreal.ai/specs/.

- Nreal light AR glasses review, Online: https://www.tomshardware.com/reviews/nreal-light-ar-smart-glasses.

# Proposal

It is proposed to include clause 2 of this contribution in the clause 5.4 of the permananent document, as sub-clause 5.4.2.