3GPP TSG-S4 Meeting #129	S4-241611

# Electronic Meeting, 19th – 23rd August 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR |  | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] RTCP congestion control feedback message for XR services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Comunications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Key issue #7: RTCP messages to better support XR services in 5G. Transport-wide RTCP feedback message discussed in S4aR240042 for providing feedback information about each packet received is not from an active IETF draft. An RTP receiver sends the RTCP CC feedback message defined in RFC 8888 to provide the status of each RTP packet received and the received media quality metrics information to an RTP sender. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding RTCP congestion control feedback message defined in IETF RFC 8888 for support of XR services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1	Background

The new SA4 Rel-19 study item on “5G Real-time Transport Protocol Configurations, Phase 2” (5G_RTP_Ph2) has been approved in SP-240523. The study item lists thirteen distinct key issues to improve 5G RTP as defined in TS 26.822. In S4aR240042, key issue #7 on RTCP messages to better support XR services in 5G has been discussed. In this contribution we propose an RTCP congestion control feedback message that can be used for XR services.

## 2	Changes

First Change

## 6.x	Solution #x: Candidate RTCP messages and RTP header extensions to support XR services in 5G

### 6.x.1	Key Issue mapping

This maps to Key Issue #7.

### 6.x.2	Description

### 6.x.2.1 	RTCP messages

To understand the RTCP messages that may be used for supporting XR applications, we need to know what RTCP messages have been used in commercial systems. For this purpose, we look at the WebRTC implementation [WebRTC-code].

RTCP messages defined in RFCs:

- Feedback messages:

  - Transport layer FeedBack messages (RTPFB): The PT value is 205.

    - Generic NACK (RFC 4585): The PT is 205 and the format type (FMT) value is 1. The Generic NACK is used to indicate RTP packet losses, identified by the means of a packet identifier and a bit mask.

    - RTP Congestion Control Feedback (CCFB) (RFC 8888): The PT value is 205 and the FMT value is 11. The congestion control feedback message is used to transmit the status of each RTP packet received at the RTP receiver along with the RTP sequence number, packet arrival time and packet Explicit Congestion Notification (ECN) Marking. This message specifies a common RTCP feedback packet format that can be used by Network-Assisted Dynamic Adaptation (NADA) [RFC 8698], Self-Clocked Rate Adaptation for Multimedia (SCReAM) [RFC 8298], Google Congestion Control [Google-GCC], and Shared Bottleneck Detection [RFC 8382] congestion control algorithms.

The format and semantics of the RTCP CCFB packet is as shown below.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| FMT=11  |   PT = 205    |          length               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 SSRC of RTCP packet sender                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   SSRC of 1st RTP Stream                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          begin_seq            |          num_reports          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R|ECN|  Arrival time offset    | ...                           .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.                                                               .

.                                                               .

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   SSRC of nth RTP Stream                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|          begin_seq            |          num_reports          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R|ECN|  Arrival time offset    | ...                           |

.                                                               .

.                                                               .

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                 Report Timestamp (32 bits)                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

  - begin_seq [16 bits]: The first sequence number that this feedback message reports on.

  - Received (R) [1 bit]:  A boolean that indicates whether the packet was received.  0 indicates that the packet was not yet received and the subsequent 15 bits (ECN and ATO) in this 16-bit packet metric block are also set to 0 and MUST be ignored. 1 indicates that the packet was received and the subsequent bits in the block need to be parsed.

  - num_reports [16 bits]: The number of RTP packets reported in this feedback message. The report block contains a 16-bit packet metric block for each RTP packet that has a sequence number in the range begin_seq to begin_seq+num_reports inclusive.

  - ECN [2 bits]: The echoed ECN mark of the packet. These bits are set to 00 if not received or if ECN is not used.

  - Arrival time offset (ATO) [13 bits]: The arrival time of the RTP packet at the receiver, as an offset before the time represented by the Report Timestamp (RTS) field of this RTCP congestion control feedback report. The ATO field is in units of 1/1024 seconds (this unit is chosen to give exact offsets from the RTS field) so, for example, an ATO value of 512 indicates that the corresponding RTP packet arrived exactly half a second before the time instant represented by the RTS field.

  - Report Timestamp (RTS) [32 bits]: This value denotes the time instant on which this packet is reporting and is the instant from which the arrival time offset values are calculated. The value of the RTS field is derived from the same clock used to generate the NTP timestamp field in RTCP Sender Report (SR) packets.

End of Changes

## 3	Proposal

We propose to agree the proposed changes in clause 2 into TR 26.822.