3GPP TSG-SA WG4 Meeting #99	S4-180710

# Roma, Italy, July 9th - 13th 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.132 | CR | 0000 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of speech quality assessment method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SPAN |  |  |  |  |  | Date: |  |  | 2018-07-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The SWB/FB speech quality prediction model B according to ETSI TS 103 281 is used for the evaluation of noise suppression quality assessment, but is not commercially available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Model B according ETSI TS 103 281 is removed from the measurement analysis |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance requirements and objectives for SWB and FB terminals cannot be evaluated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

******** START OF MODIFIED CLAUSE ********

## 9.12	Quality (speech quality, noise intrusiveness) in the presence of ambient noise

### 9.12.1	Handset

The speech quality in sending for super-wideband systems is tested based on ETSI TS 103 281 [50]. This test method leads to three MOS-LQOfb quality numbers:

-		N-MOS-LQOfb: 	Transmission quality of the background noise

-		S-MOS-LQOfb: 	Transmission quality of the speech

-		G-MOS-LQOfb:	Overall transmission quality

The test arrangement is given in clause 5.1.5. For connections with handset UE, the measurement is conducted for 8 noise conditions as described in Table 2i. The measurements should be made in the same unique and dedicated call. The noise types shall be presented according to the order specified in Table 2i.

Table 2i: Noise conditions used for ambient noise simulation in handset mode as specified in ES 202 396-1 [35]

| Description | File name | Duration | Level | Type |
| --- | --- | --- | --- | --- |
| Recording in pub | Pub_Noise_binaural_V2 | 30 s | L: 75,0 dB(A) R: 73,0 dB(A) | Binaural |
| Recording at pavement | Outside_Traffic_Road_binaural | 30 s | L: 74,9 dB(A) R: 73,9 dB(A) | Binaural |
| Recording at pavement | Outside_Traffic_Crossroads_binaural | 20 s | L: 69,1 dB(A) R: 69,6 dB(A) | Binaural |
| Recording at departure platform | Train_Station_binaural | 30 s | L: 68,2 dB(A) R: 69,8 dB(A) | Binaural |
| Recording at the drivers position | Fullsize_Car1_130Kmh_binaural | 30 s | L: 69,1 dB(A) R: 68,1 dB(A) | Binaural |
| Recording at sales counter | Cafeteria_Noise_binaural | 30 s | L: 68,4 dB(A) R: 67,3 dB(A) | Binaural |
| Recording in a cafeteria | Mensa_binaural | 22 s | L: 63,4 dB(A) R: 61,9 dB(A) | Binaural |
| Recording in business office | Work_Noise_Office_Callcenter_binaural | 30 s | L: 56,6 dB(A) R: 57,8 dB(A) | Binaural |

1)	Before starting the measurements, the calibration procedure described in clause 9.5 of ETSI TS 103 281 [50] shall be performed with the UE in handset mode. Also, a proper conditioning sequence shall be used. The conditioning sequence shall be comprised of the four additional sentences 1-4 described in ETSI TS 103 281 [50], applied to the beginning of the 16-sentence test sequence.

NOTE: The sequence of speech samples concatenated for the test signal, consisting of alternating talkers in the sending direction, reduces the overall test time but may represent an unrealistic behaviour for certain voice enhancement technologies. Alternative concatenations are for further study.

2)	The send speech signal consists of the 16 sentences of speech as described in ETSI TS 103 281 [50] The test signal level is - 1.7 dBPa at the MRP, measured as active speech level per ITU-T P.56 [37]. Two signals are required for the tests:

–	The clean speech signal is used as the undisturbed reference (see ETSI TS 103 281 [50])

–	The send signal is recorded at the POI.

3)	N-MOS-LQOfb, S-MOS-LQOfb and G-MOS-LQOfb are calculated according to the Model A objective predictor described in ETSI TS 103 281[50] on a per sentence basis and averaged over all 16 sentences. The final results are derived as follows:

S-MOS-LQOfb = S-MOS-LQOfb_modelA

N-MOS-LQOfb = 1.438*N-MOS-LQOfb_modelA – 1.959

G-MOS-LQOfb = G-MOS-LQOfb_modelA

4)	The measurement is repeated for each ambient noise condition described in Table 2i.

5)	The average of the results derived from all ambient noise types is calculated.

NOTE: Recent investigations indicated an improved prediction performance when combining both models A and B. The usage of model B according to ETSI TS 103 281 [50] is for further study, pending a commercially available implementation.

### 9.12.2	Hand-held hands-free

For connections with hand-held hands-free UE, when using the simulation method described in TS 103 224 [43], the measurement is conducted for 5 noise conditions as described in Table 2i2. When using the ES 202 396-1 method, the equivalent binaurally recorded noises described in Table 2i2, and available in the source file directory of TS 103 224 [43], are used.

Table 2i2: Noise conditions used for ambient noise simulation in hand-held hands-free mode as specified in TS 103 224 [43], A-weighted

| Name | Description | Length | Hands-free Levels | Binaural L | Binaural R |
| --- | --- | --- | --- | --- | --- |
| Full-size car 130 km/h (FullSizeCar_130) | HATS and microphone array at co-drivers position | 30 s | 1: 69,5 dB 2: 68,6 dB 3: 68,6 dB 4: 68,7 dB 5: 68,8 dB 6: 68,8 dB 7: 69,2 dB 8: 69,7 dB | 68.7 dB | 70.7 dB |
| Crossroadnoise (Crossroadnoise) | HATS and microphone array standing outside near a crossroad | 30 s | 1: 69,9 dB 2: 69,6 dB 3: 69,6 dB 4: 69,9 dB 5: 69,6 dB 6: 69,5 dB 7: 69,6 dB 8: 69,7 dB | 70.8 dB | 71.6 dB |
| Cafeteria (Cafeteria) | HATS and microphone array inside a cafeteria | 30 s | 1: 69,0 dB 2: 69,7 dB 3: 69,6 dB 4: 69,8 dB 5: 69,5 dB 6: 69,5 dB 7: 69,7 dB 8: 70,0 dB | 69.8 dB | 70.3 dB |
| Sales Counter (SalesCounter) | HATS and microphone array in a supermarket | 30 s | 1: 65,5 dB 2: 65,3 dB 3: 65,2 dB 4: 65,5 dB 5: 65,6 dB 6: 65,3 dB 7: 65,2 dB 8: 65,3 dB | 66.7 dB | 66.6 dB |
| Callcenter 2 (Callcenter) | HATS and microphone array in business office | 30 s | 1: 59,3 dB 2: 59,3 dB 3: 59,5 dB 4: 59,6 dB 5: 59,4 dB 6: 59,3 dB 7: 59,3 dB 8: 59,5 dB | 60,2 dB | 60,0 dB |

1)	Before starting the measurements, the calibration procedure described in clause 9.5 of ETSI TS 103 281 [50] shall be performed with the UE in hands-free mode. Also, a proper conditioning sequence shall be used. The conditioning sequence shall be comprised of the four additional sentences 1-4 described in ETSI TS 103 281 [50], applied to the beginning of the 16-sentence test sequence.

NOTE: The sequence of speech samples concatenated for the test signal, consisting of alternating talkers in the sending direction, reduces the overall test time but may represent an unrealistic behaviour for certain voice enhancement technologies. Alternative concatenations are for further study.

2)	The send speech signal consists of the 16 sentences of speech as described in ETSI TS 103 281 [34]. For connections with a hand-free UE, the test signal level is +1.3dBPa at the MRP, measured as active speech level according to ITU-T P.56 [37]. Two signals are required for the tests:

–	The clean speech signal is used as the undisturbed reference (see ETSI TS 103 281 [50])

–	The send signal is recorded at the POI.

3)	N-MOS-LQOfb, S-MOS-LQOfb and G-MOS-LQOfb are calculated according to the Model A objective predictor described in ETSI TS 103 281 [50] on a per sentence basis and averaged over all 16 sentences. The final results are derived as follows:

S-MOS-LQOfb = S-MOS-LQOfb_modelA

N-MOS-LQOfb = 1.438*N-MOS-LQOfb_modelA – 1.959

G-MOS-LQOfb = G-MOS-LQOfb_modelA

4)	The measurement is repeated for each ambient noise condition described in Table 2i2.

5)	The average of the results derived from all ambient noise types is calculated.

NOTE: Recent investigations indicated an improved prediction performance when combining both models A and B. The usage of model B according to ETSI TS 103 281 [50] is for further study, pending a commercially available implementation.

******** END OF MODIFIED CLAUSE ********